ENGINEERING SYSTEM SPECIFICATION

Real-Time Perception, Sensor Fusion, and Navigation Stack for AMR Platform v4.2

Document Identifier: SPEC-AMR-v4.2-REV-C | Engineering Change Order: ECO-88421
Status: APPROVED FOR PRODUCTION | Security Classification: INTERNAL ENGINEERING SPECIFICATION

――――――――――――――――――――――――――――――――――――――――――――――――――――――――――――――

ARCHITECTURAL BASELINE SUMMARY This document defines the production baseline for AMR Platform v4.2. All software components must adhere to the deterministic latency budgets established in Section 5 and interface via the CAN 2.0B message frames defined in Section 6. Firmware builds failing the HIL regression suite (Section 8) are strictly prohibited from fleet deployment.

# 1. System Scope & Operational Boundary Definitions

This engineering specification establishes the definitive hardware interfaces, state estimation mathematics, ROS 2 middleware communications, and fail-safe interlocks governing the Autonomous Mobile Robot (AMR) Platform v4.2 chassis. The system is designed for high-density logistics facilities, automated pallet transport, and dynamic human-robot collaborative work cells.

Operational Design Domain (ODD) Constraints:

Nominal Payload Capacity: 500.0 kg continuous rated load (650.0 kg structural peak limit).

Maximum Linear Velocity: 2.2 m/s forward / reverse along designated high-speed transit corridors.

Maximum Angular Velocity: 1.57 rad/s (90 deg/s) during zero-radius differential pivot maneuvers.

Surface Grading & Ingress: Ingress Protection rating IP54; maximum floor slope gradient of 4.5 degrees.

Ambient Operating Temperature: -5°C to +48°C continuous at non-condensing relative humidity <= 90%.

# 2. Sensor Suite Hardware Interconnect Architecture

The perception stack combines synchronized active optical sensors, high-frequency inertial measurement units, and dual wheel encoders to achieve sub-centimeter localization fidelity under varying lighting and structural clutter.

| Sensor Subsystem | Physical Model / Part | Bus Interface | Sample Rate | Operational Role |
| --- | --- | --- | --- | --- |
| Primary Safety LiDAR | SICK microScan3 Core | 100BASE-TX Ethernet | 33 Hz (30ms) | Certified safety field zone protective stop |
| Secondary Nav LiDAR | Hokuyo UST-20LX | 100BASE-T Ethernet | 40 Hz (25ms) | Natural feature SLAM & obstacle bypass |
| Front 3D Depth Camera | Intel RealSense D435i | USB 3.1 Gen 1 | 30 FPS | Negative obstacle / forklift tines detection |
| Tactical 6-DOF IMU | Analog Devices ADIS16470 | SPI / Dedicated MCU | 1000 Hz | High-frequency dead-reckoning state propagation |
| Wheel Encoders | Broadcom HEDS-5540 | Quadrature / FPGA | 2000 Hz | Differential wheel velocity feedback |

# 3. Multi-Rate State Estimation: Error-State Extended Kalman Filter (ES-EKF)

State estimation is implemented via an Error-State Extended Kalman Filter (ES-EKF) executing on the real-time compute subsystem. By partitioning state representation into a nominal high-rate state and a zero-mean error state, numerical instabilities associated with attitude quaternion singularities are fully suppressed.

Nominal State Vector Definition:

x = [p_x, p_y, p_z, v_x, v_y, v_z, q_w, q_x, q_y, q_z, b_gx, b_gy, b_gz, b_ax, b_ay, b_az]^T

Where p represents cartesian position in map frame, v represents translational velocity in base frame, q represents unit rotation quaternion, and b_g, b_a denote gyroscope and accelerometer bias vectors respectively.

Propagation Stage (1000 Hz): IMU accelerometer and gyroscope specific force integration drives nominal state forward via fourth-order Runge-Kutta discretization.

Correction Stage 1 (100 Hz): Wheel encoder forward kinematics provide non-holonomic velocity constraints (v_y = 0 assumption with empirical slip variance scaling).

Correction Stage 2 (20 Hz): 2D LiDAR scan-matching pose estimates (Normal Distributions Transform - NDT) provide absolute metric positioning corrections against pre-computed static occupancy grids.

Outlier Rejection Gating: All observation innovations must satisfy Chi-Square gating: d_M^2 = r_k^T * S_k^(-1) * r_k <= gamma (where gamma = 9.21 for 2-DOF p=0.01 significance).

# 4. ROS 2 Middleware Topology & Quality of Service (QoS)

The robotic software stack is architected on ROS 2 Humble Hawksbill, executed on an Ubuntu 22.04 LTS kernel patched with PREEMPT_RT. Inter-process communication between high-bandwidth perception nodes utilizes zero-copy Eclipse iceoryx shared memory bindings.

| Topic Namespace | Message Type | Reliability | Durability | Transport Layer |
| --- | --- | --- | --- | --- |
| /sensors/lidar_front/scan | sensor_msgs/LaserScan | BEST_EFFORT | VOLATILE | iceoryx Zero-Copy Shared Mem |
| /localization/odom_filtered | nav_msgs/Odometry | RELIABLE | VOLATILE | CycloneDDS UDP Loopback |
| /planning/global_plan | nav_msgs/Path | RELIABLE | TRANSIENT_LOCAL | CycloneDDS UDP Loopback |
| /control/cmd_vel | geometry_msgs/Twist | RELIABLE | VOLATILE | Shared Memory IPC |
| /safety/interlock_state | amr_msgs/SafetyState | RELIABLE | TRANSIENT_LOCAL | Deterministic CAN Bus Bridge |

# 5. Real-Time Compute Budget & Execution Timing Deadlines

To guarantee deterministic cycle times under worst-case execution conditions (WCET), hardware threads are isolated via Linux CPU core affinity (cgroups) on the 12-core NVIDIA Jetson AGX Orin Industrial compute module:

Cores 0-3 (Perception / Deep Inference): TensorRT INT8 YOLOv8 3D obstacle inference. Budget: 22.0 ms per frame @ 30 Hz.

Cores 4-5 (State Estimation / EKF): High-priority RT-FIFO (priority 85). Budget: 2.8 ms per cycle @ 100 Hz.

Cores 6-7 (Trajectory Planning / Costmaps): Timed-Elastic-Band (TEB) optimization. Budget: 42.0 ms per cycle @ 10 Hz.

Cores 8-9 (CAN Bus Communication & Motor Feedback): RT-FIFO (priority 90). Budget: 1.5 ms per cycle @ 100 Hz.

Cores 10-11 (Fleet Telemetry & Background Logging): Standard CFS Linux scheduling for non-critical logging.

# 6. Interface Control Document (ICD): CAN Bus 2.0B Specification

Low-level actuator control, motor telemetry, and battery management system (BMS) communications interface over an isolated CAN 2.0B bus operating at 1.0 Mbps baud rate with 11-bit standard arbitration identifiers.

| CAN ID (Hex) | Periodicity | Data Length | Signal Payload Architecture |
| --- | --- | --- | --- |
| 0x100 | 10 ms (100 Hz) | 8 Bytes | Byte 0-3: Linear Vel Target (float32, mm/s) Byte 4-7: Angular Vel Target (float32, mrad/s) |
| 0x180 | 20 ms (50 Hz) | 4 Bytes | Byte 0: Safety Relay State (0=Trip, 1=Armed) Byte 1: E-Stop State (0xAA=OK, 0x55=Depressed) Byte 2: Bumper Switch Mask Byte 3: Heartbeat Counter (0-255) |
| 0x200 | 100 ms (10 Hz) | 8 Bytes | Byte 0-1: Battery Pack Voltage (uint16, 0.1V/bit) Byte 2-3: Pack Current (int16, 0.1A/bit) Byte 4: State of Charge SOC (0-100%) Byte 5: Max Cell Temp (°C) |
| 0x280 | 10 ms (100 Hz) | 8 Bytes | Byte 0-3: Left Wheel Position (int32 ticks) Byte 4-7: Right Wheel Position (int32 ticks) |

# 7. Deterministic Safety State Machine & Fault Isolation

The robotic chassis implements an IEC 61508 SIL-2 compliant deterministic finite state machine (FSM). Transition triggers are verified via dual-redundant safety relays prior to motor drive bridge energization:

STATE_INIT: Boot hardware, calibrate IMU zero-point bias, establish CAN bus sync. Duration <= 4.0s.

STATE_LOCALIZING: Scan matching against reference occupancy grid. Convergence criteria: covariance trace Tr(P) < 0.05 m^2.

STATE_MISSION_ACTIVE: Velocity trajectory tracking engaged. Dynamic safety fields active.

STATE_OBSTACLE_SLOWDOWN: Transient entity detected within yellow warning field (distance 1.5m - 2.5m). Velocity clamped to 0.4 m/s.

STATE_SAFETY_STOP: Protective field breach (<1.2m). Electromagnetic brake coil de-energized, deceleration >= 2.5 m/s^2.

STATE_EMERGENCY_LOCKOUT: Hardware E-Stop button depressed or communication watchdog timeout (>150ms). Requires manual reset.

# 8. Hardware-in-the-Loop (HIL) Verification & Test Matrix

Prior to production firmware deployment, all release artifacts must pass automated HIL test execution. Criteria and passing thresholds are summarized below:

| Verification ID | Test Scenario | Acceptance Criterion | Test Artifact |
| --- | --- | --- | --- |
| VER-HIL-01 | Synthetic Point-Cloud Rain / Dust Injection | Zero false-positive safety stops over 100 simulated operating hours | Automated Gazebo/Isaac HIL Sim |
| VER-HIL-02 | Simulated CAN Bus Packet Corruption (CRC error) | Controller transition to STATE_SAFETY_STOP within <= 35.0 milliseconds | Vector CANoe Diagnostic Harness |
| VER-HIL-03 | Wheel Traction Loss (Wet Epoxy Floor Sim) | ES-EKF localization error <= 0.08 meters over 25 meter transit run | Dual-axis Roller Dynamometer Rig |
| VER-HIL-04 | Pedestrian Cut-In at 1.8 m/s Forward Speed | Complete stop achieved with minimum 0.35 meter safety buffer preserved | Servo-actuated Dummy Target Track |